- FLEX catalog 2023

  - 0 plot

  - 1 construction preparation

  - 2.1 Building shell

    - C General information about the shell of buildings

    - D Shell up to the top edge of the floor slab

    - E Shell of building above floor slab

      - E0 ceilings, stairs and balconies

      - E1 roofs

      - E2 supports

      - E3 external walls to basement floors

      - E4 external walls to ground/upper floors

      - E5 windows, external doors and gates

      - E6 interior walls (shell)

      - E7 Additional services Shell construction

  - 2.2 Building technology

  - 2.3 Building expansion

  - 2.4 Building, other

- 3 Structural operating facility

- 4 Operating equipment

- 5 equipment

- 6 Miscellaneous

- 7 environment

- 8 Other costs

Code:

Full text search:   The title and description of the component are searched using an OR link. You can search with one or more whole words or with parts of words.

E Shell of building above floor slab

| E0 | FLEX E0-001 | Blanket raw (flex) |
| --- | --- | --- |
| E0 | FLEX E0-001-001 | Concrete ceiling d 24cm / Fe 80kg/m3 |
| E0 | FLEX E0-001-002 | Concrete ceiling d 24cm / Fe 90kg/m3 |
| E0 | FLEX E0-001-003 | Concrete ceiling d 24cm / Fe 100kg/m3 |
| E0 | FLEX E0-002 | Ceiling with underlay (flex) |
| E0 | FLEX E0-002-001 | Concrete ceiling d=16cm (Fe 80kg/m3) with underlayment |
| E0 | FLEX E0-002-002 | Concrete ceiling d=20cm (Fe 80kg/m3) with underlayment |
| E0 | FLEX E0-002-003 | Concrete ceiling d=26cm (Fe 80kg/m3) with underlayment |
| E0 | FLEX E0-002-010 | Solid wood ceiling (BSH) d 16cm with underlay |
| E0 | FLEX E0-003 | Ceiling against unheated with thermal insulation and underlayment (flex) |
| E0 | FLEX E0-003-001 | Concrete ceiling 18 cm against unheated with EPS thermal insulation 12 cm U 0.23 W/m²K |
| E0 | FLEX E0-003-010 | Solid wood ceiling 16 cm against unheated with glass wool insulation 10 cm U 0.24 W/m²K |
| E0 | FLEX E0-005 | Ceiling (floor) against unheated with thermal insulation on the warm side (flex) |
| E0 | FLEX E0-005-001 | Concrete ceiling d=24cm (Fe 80kg/m3) against unheated with glass wool insulation 14cm warm side U 0.25 W/m²K |
| E0 | FLEX E0-005-002 | Concrete ceiling d=24cm (Fe 80kg/m3) against unheated with rock wool insulation on the warm side U 0.25 W/m²K |
| E0 | FLEX E0-005-003 | Concrete ceiling d=24cm (Fe 80kg/m3) against unheated with EPS insulation on the warm side U 0.24 W/m²K |
| E1 | FLEX E1-001 | Pitched roof with insulation in the supporting structure (flex) |
| E1 | FLEX E1-001-001 | Pitched roof with glass wool insulation 24cm in the supporting structure U 0.17 W/m²K |
| E1 | FLEX E1-001-002 | Pitched roof with insulation ISOCONFORT 032, 20cm in the supporting structure U 0.17 W/m²K |
| E1 | FLEX E1-002 | Pitched roof with insulation in and below the supporting structure (flex) |
| E1 | FLEX E1-002-001 | Pitched roof with rock wool insulation in and below the supporting structure U 0.17 W/m²K |
| E1 | FLEX E1-003 | Pitched roof with insulation in and above the supporting structure (flex) |
| E1 | FLEX E1-004 | Flat roof solid construction (compact roof) with greenery (flex) |
| E1 | FLEX E1-004-001 | Flat roof concrete 16 cm (compact roof) EPS 22cm green U 0.17 W/m²K |
| E1 | FLEX E1-004-002 | Flat roof concrete 16 cm (compact roof) PUR/PIR 18cm green U 0.17 W/m²K |
| E1 | FLEX E1-004-003 | Flat roof concrete 16 cm (compact roof) foam glass 24cm green U 0.17 W/m²K |
| E1 | FLEX E1-004-004 | Flat roof concrete 16 cm (compact roof) Flumroc insulation board PRIMA 22cm green U 0.17 W/m²K |
| E1 | FLEX E1-004-005 | Flat roof concrete 16cm (compact roof) swissporLAMBDA Roof 16cm green U 0.17 W/m²K |
| E2 | FLEX E2-001 | Support (flex) |
| E2 | FLEX E2-001-001 | Clad steel support HEA 200 (R60) |
| E3 | FLEX E3-001 | External wall against ground (flex) |
| E3 | FLEX E3-001-001 | Concrete outer wall 25cm from the ground |
| E3 | FLEX E3-002 | External wall against the ground with external insulation (flex) |
| E3 | FLEX E3-002-001 | Concrete wall against soil 25cm with external insulation XPS 20cm U 0.17 W/m²K |
| E3 | FLEX E3-002-002 | Concrete wall against ground 25cm with external insulation PUR/PIR 18cm U 0.17 W/m²K |
| E3 | FLEX E3-003 | External wall against the ground with glued internal insulation (flex) |
| E3 | FLEX E3-003-001 | Concrete external wall 25 cm against the ground with glued XPS internal insulation U 0.17 W/m²K |
| E3 | FLEX E3-004 | External wall against the ground with internal insulation inhomogeneous (flex) |
| E3 | FLEX E3-004-001 | Concrete external wall 25cm against the ground with glass wool internal insulation inhomogeneous (10+12cm) U 0.17 W/m²K |
| E4 | FLEX E4-001 | Exterior wall (flex) |
| E4 | FLEX E4-001-001 | Exterior wall Ytong Thermobloc PL-07, 40cm U 0.17 W/m2K |
| E4 | FLEX E4-002 | Exterior wall compact facade (flex) |
| E4 | FLEX E4-002-001 | External wall BN 18cm, EPS insulation 20cm U 0.17 W/m²K |
| E4 | FLEX E4-003 | Exterior wall, ventilated facade (flex) |
| E4 | FLEX E4-003-001 | Exterior wall, ventilated facade / BN 20cm / glass wool 24cm / wooden facade U 0.16 W/m²K |
| E4 | FLEX E4-003-002 | Exterior wall, ventilated facade / BN 20cm / ISOVER PB F 030 18cm / wooden facade U 0.16 W/m²K |
| E4 | FLEX E4-004 | Exterior wall double-shell masonry (flex) |
| E4 | FLEX E4-004-001 | External wall BN 15cm / KS 12cm / 18cm Flumroc insulation board 1 / U 0.17 W/m²K |
| E4 | FLEX E4-004-002 | External wall BN 15cm / KS 12cm / 16cm ISOVER PB F 030 / U 0.16 W/m²K |
| E4 | FLEX E4-004-003 | External wall BN 15cm / KS 12cm / 18cm swissporEPS 30 / U 0.16 W/m²K |
| E5 | FLEX E5-001 | window (flex) |
| E5 | FLEX E5-002 | Exterior doors (flex) |
| E6 | FLEX E6-001 | Interior exposed brickwork flex) |
| E6 | FLEX E6-001-001 | Exposed concrete wall d15cm |
| E6 | FLEX E6-001-002 | Interior exposed brickwork KS 15cm |
| E6 | FLEX E6-003 | Interior masonry plastered (flex) |
| E6 | FLEX E6-003-001 | Brick masonry 15cm plastered |
| E6 | FLEX E6-004 | Internal wall against unheated with internal insulation (flex) |
| E6 | FLEX E6-005 | Internal wall against unheated with external insulation (flex) |
| M1 | FLEX M1-001 | Lightweight stud wall (flex) |
| M1 | FLEX M1-001-001 | Lightweight stud wall 12.5 / 75 / 12.5mm h 3.0m |
| M1 | FLEX M1-002 | Lightweight stud wall (wood) (flex) |
| M1 | FLEX M1-002-001 | Wooden stud wall 12.5 / 80 / 12.5mm / h3.0m |
| M1 | FLEX M1-005 | Interior door (flex) |
| M1 | FLEX M1-005-001 | Room door 2.0x0.9m, wood, glass insert, steel frame |
| M3 | FLEX M3-001 | Floor coverings (flex) |
| M3 | FLEX M3-001-001 | Floor covering artificial stone slab including anhydrite underlay 6cm |
| M3 | FLEX M3-002 | Floating floor coverings (flex) |
| M3 | FLEX M3-002-001 | Floor covering parquet 3-layer 15mm including anhydrite underlay 6cm |
| M7 | FLEX M7-001 | Kitchen fittings and furniture (flex) |